Mr. SQUID User’s Guide

This manual is written in reference to main standard manual.

Step 1: Check both the channel of oscilloscope is DC Coupled (See last page for this)

Step 2: Follow section 3.1 (Page 6) setting up Mr. SQUID system all 6 points.

Step 3: Your Instrument is ready to use.

Observe graph as in section 3.6 this is your target 1.

Use these 5 knobs to set the system

- Flux Offset

- Bias Offset

- Sweep Output

- 2 Sensitivity knobs in oscilloscope

Step 4: Click a picture of complete screen once you are set. Make sure you get exact same curve as in section 3.6

Step 5: Set the electronics to V-Φ and observe graph in section 3.7. This is your target 2.

# Hint/Optional: Set channel Y to AC coupled.

Step 6: Use 7 knobs to set the curve

1 Previous 5 knobs

2 Two knobs to set the horizontal and vertical position

Step 7:  Read page 12, 13, 14 for V-I

Read page 15, 16,17,18,19 for V-Φ

Do all the calculations given.

Calculation Help:

Section 3.8

βL = 2IcL/ Φ0

Ic – Fig 3.6 & trailing notes

L – Eq. 3.3

Φ0 – Period from graph fig 3-9

βL = 4IcRN/ π∆V -1

RN – Fig 3-7 & trailing notes

∆V – Peak to peak voltage gap Fig. 3-9

Precaution:

- Use the probe carefully. Do not damage it.

How to take out the probe:

Be ready with 1 hair dryer (Use hair dryer in speed 1), 1 tissue roll

Take out probe, constantly blow air uniformly.